Technical Document

Document Name: Upgrade Software Operation Instruction

Contents

1．Objective	3

2．Operation Instruction	3

3．Notes	6

# 1．Objective

This tool is only be run in PC with Windows® XP or Windows® 7.

This tool is only applicable to MF97V .

Before using this tool, please make sure installed device drivers.

In Windows 7system, you must select the administrator permissions to open the tool.

# 2．Operation Instruction

- Tool Main interface:

In windows xp system,you can double-click on the tool icon to open the tool.But in windows 7 system,you must select the administrator permissions to open the tool.

The tool main interface as below:

- Driver install:

Before download ,you must install drive of MF97V device.

- Enter FTM:

Enter FTM mode.Under shutdown status, and then hold the device volume down key and press the power button , when the word "FTM" is shown on the screen, release the volume down key. and then use the data cable connect device and computer.

Then enters the device manager, check the port:

- Port config:

Click, According to the [ device port management] tool [COM] port corresponding [USB] port,choose menu “Port config”, fill the [USB] port and click “OK”.

Then you can see the device information.

- Software directory:

Click the button, choose the folder which contains your software version.

.

- Download:

Click button  or  to start download.

- CSV Log:

Click menu  “OPTION” ->“EXPORT CSV” button to export CSV log file, you can open the csv file with Office Excel program or plain text editor tool.

- Counter:

The counter useds for the number of times and the success number that displays consumer to carry on a download.

Current time:The current time of the system.

Total amount:The procedure installs up to now, carry on of total succeed to count/total download number of times.

Current amount:Last time pure null up to now carry on of total succeed to count/total download number of times.

Last cleartime:Carries on the time that the pure null operate last time.

The consumer will hint after click the Clear button whether carry on pure null, if select BE, current scaling will drive pure empty.

# 3．Notes

- Please in the usage originally the software is previous, install right driver.

- In Windows 7system, you must select the administrator permissions to open the tool.

- When this software circulates each time, is quasi- to circulate an example with solid procedure, can't open procedure for many times.

- Please backup your contacts and personal information before upgrade.

- Only eject device when five device successful all，Be sure you have power supply during the upgrade.

- Before download,please pull out the SIM card.

Notes: if you have any question,please visit

http://support.zte.com.cn